snmptrad

snmptrapd script

#!/bin/bash

# ucd-snmp init file for snmptrapd

#

# chkconfig: - 50 50

# description: Simple Network Management Protocol (SNMP) Trap Daemon

#

# processname: /usr/sbin/snmptrapd

# config: /etc/snmp/snmptrapd.conf

# config: /usr/share/snmp/snmptrapd.conf

# pidfile: /var/run/snmptrapd.pid

#

# source function library

. /etc/init.d/functions

OPTIONS="-s -c /etc/snmp/snmptrapd.conf -C  -u /var/run/snmptrapd.pid"

RETVAL=0

prog="snmptrapd"

start() {

echo -n $"Starting $prog: "

daemon /usr/sbin/snmptrapd $OPTIONS

RETVAL=$?

echo

touch /var/lock/subsys/snmptrapd

return $RETVAL

}

stop() {

echo -n $"Stopping $prog: "

killproc /usr/sbin/snmptrapd

RETVAL=$?

echo

rm -f /var/lock/subsys/snmptrapd

return $RETVAL

}

reload(){

stop

start

}

restart(){

stop

start

}

condrestart(){

[ -e /var/lock/subsys/snmptrapd ] && restart

return 0

}

case "$1" in

start)

start

;;

stop)

stop

;;

restart)

restart

;;

reload)

reload

;;

condrestart)

condrestart

;;

status)

status snmptrapd

RETVAL=$?

;;

*)

echo $"Usage: $0 {start|stop|status|restart|condrestart|reload}"

RETVAL=1

esac

exit $RETVAL

snmptrapd.conf

# /etc/snmp/snmptrapd.conf

traphandle SNMPv2-MIB::coldStart /usr/local/nagios/libexec/eventhandlers/handle-cisco-traps 1

traphandle SNMPv2-MIB::warmStart /usr/local/nagios/libexec/eventhandlers/handle-cisco-traps 2

traphandle IF-MIB::linkDown /usr/local/nagios/libexec/eventhandlers/handle-cisco-traps 3

traphandle IF-MIB::linkUp /usr/local/nagios/libexec/eventhandlers/handle-cisco-traps 4

traphandle default /usr/local/nagios/libexec/eventhandlers/handle-cisco-traps 5

/usr/local/nagios/libexec/eventhandlers/handle-cisco-traps

handle-cisco-traps

#!/bin/sh

#

# This script should handle the traps it will receive

# from the snmptrapd and defined in the snmptrapd.conf

#

read host

hostname="unkown"

ip=`host $host | awk -F" " '{print $3}'

esac

state=-1

output="No Output"

case $1 in

1) output="Critical: Coldstart"

state=2

;;

2) output="Warning: Warmstart"

state=1

;;

3) output="Critical: Link down"

state=2

;;

4) output="Warning: Link up"

state=2

;;

5) output="Warning: Unknown trap"

state=1

# SNMP Trap is the name of the service

/usr/lib/nagios/plugins/eventhandlers/submit_check_result $hostname "SNMP Trap" $state "$output"

submit_check_result (Nagios)

#!/bin/sh

# SUBMIT_CHECK_RESULT

# Written by Ethan Galstad (nagios@nagios.org)

# Last Modified: 02-18-2002

#

# This script will write a command to the Nagios command

# file to cause Nagios to process a passive service check

# result.  Note: This script is intended to be run on the

# same host that is running Nagios.  If you want to

# submit passive check results from a remote machine, look

# at using the nsca addon.

#

# Arguments:

#  $1 = host_name (Short name of host that the service is

#       associated with)

#  $2 = svc_description (Description of the service)

#  $3 = return_code (An integer that determines the state

#       of the service check, 0=OK, 1=WARNING, 2=CRITICAL,

#       3=UNKNOWN).

#  $4 = plugin_output (A text string that should be used

#       as the plugin output for the service check)

#

echocmd="/bin/echo"

CommandFile="/usr/local/nagios/var/rw/nagios.cmd"

# get the current date/time in seconds since UNIX epoch

datetime=`date +%s`

# create the command line to add to the command file

cmdline="[$datetime] PROCESS_SERVICE_CHECK_RESULT;$1;$2;$3;$4"

# append the command to the end of the command file

`$echocmd $cmdline >> $CommandFile`

#`$echocmd $cmdline >> /backup/snmp-trap-test.log`

Sniffer Capture Packet

Frame 1 (211 bytes on wire, 211 bytes captured)

Ethernet II, Src: 00:0e:d7:74:cc:d1, Dst: 00:b0:d0:32:7a:7c

Internet Protocol, Src Addr: 192.168.24.60 (192.168.24.60), Dst Addr: 192.168.24.58 (192.168.24.58)

User Datagram Protocol, Src Port: 53914 (53914), Dst Port: snmptrap (162)

Simple Network Management Protocol

Version: 2C (1)

Community: test

PDU type: TRAP-V2 (7)

Request Id: 0x00000002

Error Status: NO ERROR (0)

Error Index: 0

Object identifier 1: 1.3.6.1.2.1.1.3.0 (iso.3.6.1.2.1.1.3.0)

Value: Timeticks: (212951) 0:35:29.51

Object identifier 2: 1.3.6.1.6.3.1.1.4.1.0 (iso.3.6.1.6.3.1.1.4.1.0)

Value: OID: iso.3.6.1.6.3.1.1.5.3

Object identifier 3: 1.3.6.1.2.1.2.2.1.1.1 (iso.3.6.1.2.1.2.2.1.1.1)

Value: INTEGER: 1

Object identifier 4: 1.3.6.1.2.1.2.2.1.2.1 (iso.3.6.1.2.1.2.2.1.2.1)

Value: STRING: "Ethernet0"

Object identifier 5: 1.3.6.1.2.1.2.2.1.3.1 (iso.3.6.1.2.1.2.2.1.3.1)

Value: INTEGER: 6

Object identifier 6: 1.3.6.1.4.1.9.2.2.1.1.20.1 (iso.3.6.1.4.1.9.2.2.1.1.20.1)

Value: STRING: "administratively down"